| PROJECT NAME: |  |  | DRAWING NO. |  |  |  |  | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | QUESTIONS |  |  | ORIGINATOR |  |  | CHECKER |  |  |  |
|  |  |  |  | N/A | YES | NO | N/A |  | YES | NO |
| Preparation and Checking |  |  |  |  |  |  |  |  |  |  |
| 01 | Does the drawing comply with applicable codes, standards and regulatory requirements? |  |  |  |  |  |  |  |  |  |
| 02 | Does the drawing comply with applicable Project Design Criteria, system or structural functional requirements, Scope Book, and Design Basis Documents considered? |  |  |  |  |  |  |  |  |  |
| 03 | Is the coordinate system and the site arrow direction (North) on drawing correct? |  |  |  |  |  |  |  |  |  |
| 04 | Does the Allowable Bearing Capacity (ABC) and settlement on the drawing comply with geotechnical report? |  |  |  |  |  |  |  |  |  |
| 05 | Does drawing specify the footing schedule of all foundations? |  |  |  |  |  |  |  |  |  |
| 06 | Does footing schedule specify the sizes, depths, foundation levels as a minimum? |  |  |  |  |  |  |  |  |  |
| 07 | Does concrete cover to reinforcement comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 08 | Does the specified compressive strength of in-situ & precast concrete, for foundations, comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 09 | Does the specified compressive strength of concrete in grade beams comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 10 | Does the specified compressive strength of blinding concrete comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 11 | Does the specified type and compressive strength of non-shrink grout comply with Project Specifications and supplier requirement? |  |  |  |  |  |  |  |  |  |
| 12 | Does the specified compressive strength of screed comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 13 | Does the grade of Reinforcement steel comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 14 | Does the drawing specify a note for the separation of two dissimilar metals to avoid corrosion? |  |  |  |  |  |  |  |  |  |
| 15 | Does the splice length of tension splice comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 16 | Does the dowel length (of rebars) for the column include an allowance for kicker? |  |  |  |  |  |  |  |  |  |
| 17 | Has the clash between column rebars, supplementary reinforcement and anchor bolts been checked? |  |  |  |  |  |  |  |  |  |
| 18 | Have the foundation been checked for the openings/penetrations/pockets due to MEP? |  |  |  |  |  |  |  |  |  |
| 19 | Does the concrete coating/protection system comply with Project Specifications? |  |  |  |  |  |  |  |  |  |
| 20 | Has the clash between foundations and embedded pits been checked? |  |  |  |  |  |  |  |  |  |
| 21 | Has the location/coordinates/projection of embeds (anchor bolts, steel plates, sections etc.) been checked with other disciplines drawings? |  |  |  |  |  |  |  |  |  |
| 22 | Has the congestion of reinforcement at the junction of plinth beam and column been checked? |  |  |  |  |  |  |  |  |  |
| 23 | Does the backfill around the structure comply with Project Specification? |  |  |  |  |  |  |  |  |  |
| 24 | If applicable, has the dewatering note been included? |  |  |  |  |  |  |  |  |  |
| 25 | Has the drawing been checked for applicable “Holds”? |  |  |  |  |  |  |  |  |  |
| 26 | Does the Foundation Plan and Elevations show adjacent foundations or structures those require right construction sequence to avoid undermining, shoring or risk to adjacent foundation/structures? |  |  |  |  |  |  |  |  |  |
| 27 | Is there a reference on curing process to be followed? |  |  |  |  |  |  |  |  |  |
| Co-related Documents |  |  |  |  |  |  |  |  |  |  |
| 28 | Has the reference for General Notes’ drawing been provided? |  |  |  |  |  |  |  |  |  |
| 29 | Have the references of interface drawings (MEP & Architecture) been provided? |  |  |  |  |  |  |  |  |  |
| 30 | Has reference of list of drawings been provided? |  |  |  |  |  |  |  |  |  |
| Coordination and Review |  |  |  |  |  |  |  |  |  |  |
| 31 | Has appropriate interdisciplinary and intradepartmental coordination been done? |  |  |  |  |  |  |  |  |  |
| 32 | Have reviewers from other disciplines/departments evaluated items pertinent to their area and provided their comments? |  |  |  |  |  |  |  |  |  |
| 33 | Has the Responsible engineer resolved their comments? |  |  |  |  |  |  |  |  |  |
| 34 | If applicable has the drawing been stamped by a registered professional engineer? |  |  |  |  |  |  |  |  |  |
| 35 | Has the drawing been checked for constructability? |  |  |  |  |  |  |  |  |  |
| Administrative |  |  |  |  |  |  |  |  |  |  |
| 36 | Are the Drawing number and template in accordance with the project requirements? |  |  |  |  |  |  |  |  |  |
| 37 | If revised, have revisions been clearly identified with clouds and clear statement in revision block? |  |  |  |  |  |  |  |  |  |
| 38 | Have previous revision indications been removed? |  |  |  |  |  |  |  |  |  |
| No. | Checker’s Comments | Resolution |  |  |  |  |  |  |  |  |
| Originator's Name / Signature and Date: |  | Checker's Name / Signature and Date: |  |  |  |  |  |  |  |  |