74 Point Elevators Safety Checklist

| No | Hazard | EN81-80 Clause | No | Hazard | EN81-80 Clause |
| --- | --- | --- | --- | --- | --- |
| 1 | Presence of harmful materials | 5.1.4 | 40 | Car without doors | 5.8.3 |
| 2 | No or limited accessibility for disabled persons | 5.2.1 | 41 | Unsafe locking of car roof trap door | 5.8.4 |
| 3 | Drive system with poor stopping/levelling accuracy | 5.2.2 | 42 | Insufficient strength of car roof | 5.8.5 |
| 4 | No or inadequate vandal resistance | 5.3 | 43 | No or inadequate balustrade on car roof (voids) | 5.8.6 |
| 5 | No or inadequate control functions in case of fire | 5.4 | 44 | Insufficient ventilation in car | 5.8.7 |
| 6 | Well enclosures with perforate walls | 5.5.1.1 | 45 | Inadequate lighting in car | 5.8.8.1 |
| 7 | Partially enclosed well with too low enclosure | 5.5.1.2 | 46 | No or inadequate emergency lighting in car | 5.8.8.2 |
| 8 | Inadequate locking devices on access doors to well and pit | 5.5.2 | 47 | No or inadequate protection means on sheaves, pulleys and sprockets against injury | 5.9.1 |
| 9 | Inadequate vertical surface below landing door sills | 5.5.3 | 48 | No or inadequate protection against rope/chains leaving the sheaves, pulleys or sprockets | 5.9.1 |
| 10 | Counterweight/balancing weight without safety gear in case of accessible spaces below well | 5.5.4 | 49 | No or inadequate protection on sheaves, pulleys or sprockets against introduction of objects | 5.9.1 |
| 11 | No or inadequate partition of counterweight balancing weight travel path at the lowest terminal | 5.5.5 | 50 | No or inadequate safety gear and/or overspeed governor on electric elevators | 5.9.2 |
| 12 | No or inadequate pit screen for several elevators in the same well | 5.5.6.1 | 51 | No or inadequate slack rope switch for governor rope | 5.9.3 |
| 13 | No or inadequate partition for several elevators in the same well | 5.5.6.2 | 52 | No protection means against ascending car overspeed | 5.9.4 |
| 14 | Insufficient safety spaces in headroom and pit | 5.5.7 | 53 | Inadequate lift machine design for preventing uncontrolled up or down movement of the car whilst the doors are open | 5.9.4 5.12.1 |
| 15 | Unsafe pit access | 5.5.8 | 54 | No or inadequate protection against free fall, Overspeed and creeping on hydraulic elevators | 5.9.5 |
| 16 | No or inadequate isolation devices in the pit or in the pulley room | 5.5.9 | 55 | Unsuitable guidance system for counterweight or balancing weight | 5.10.1 |
| 17 | No or inadequate lighting of the well | 5.5.10 | 56 | No or inadequate buffers | 5.10.2 |
| 18 | No alarm system in pit and on car roof | 5.5.11 | 57 | No or inadequate final limit switches | 5.10.3 |
| 19 | No or unsafe means of access to machine and pulley room | 5.6.1 | 58 | Large gap between car and wall facing the car entrance | 5.11.1 |
| 20 | Slippery floor in machine or pulley room | 5.6.2 | 59 | Excessive distance between car door and landing door | 5.11.2 |
| 21 | Insufficient clearances in machine room | 5.6.3 | 60 | No or inadequate emergency operation instruction manual for rescue of entrapped passengers | 5.12.2 |
| 22 | No or inadequate protection on different levels in machine room | 5.6.4 | 61 | No hydraulic shut-off valve | 5.12.3 |
| 23 | Inadequate lighting in machine or pulley room | 5.6.5 | 62 | No independent starting/stopping machine contactors | 5.12.4 |
| 24 | Inadequate means of handling equipment | 5.6.6 | 63 | No or inadequate slack rope/chain device | 5.12.5 |
| 25 | Perforate landing doors and car doors | 5.7.1 | 64 | No run-time limiter | 5.12.6 |
| 26 | Inadequate design of landing door fixings | 5.7.2 | 65 | No or inadequate low-pressure device | 5.12.7 |
| 27 | Inappropriate glass in doors | 5.7.3 | 66 | Insufficient protection against electric shock and/or marking of electrical equipment | 5.13.1 |
| 28 | No or inadequate protection against dragging of fingers on sliding car or landing doors with glass | 5.7.4 | 67 | No or inadequate protection of lift machine electrical overload/temperature | 5.13.2 |
| 29 | No or inadequate lighting on landing | 5.7.5 | 68 | No lockable main switch | 5.13.3 |
| 30 | No or inadequate protective devices on power operated doors | 5.7.6 | 69 | No protection against phase reversal | 5.14.1 |
| 31 | Unsafe locking device of landing door | 5.7.7 | 70 | No or inadequate inspection control station and stopping device on car roof | 5.14.2 |
| 32 | Unlocking of landing door without a special tool | 5.7.8.1 | 71 | No or inadequate alarm device | 5.14.3 |
| 33 | Well enclosure with perforate walls near door locks | 5.7.8.2 | 72 | No or inadequate communication system between machine room and car (travel height ≥ 30m) | 5.14.4 |
| 34 | No automatic closing device on sliding doors | 5.7.9 | 73 | No or inadequate load or overload control on car | 5.14.5 |
| 35 | Inadequate link between panels of landing doors | 5.7.10 | 74 | Missing notices, markings and operating instructions | 5.15 |
| 36 | Inadequate fire resistance of landing doors | 5.7.11 |
| 37 | Car door moving with open landing door | 5.7.12 |
| 38 | Large car area in relation to rated load | 5.8.1 |
| 39 | Inadequate length of car apron | 5.8.2 |