| Healthcare Facility Name: |  |  |  | Reference #: | REV: 00A |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Instrumentation PPM Schedule – Healthcare |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |
| Item | Part | FQ | Action | Notes | Checked |  |  |
|  |  |  |  |  | N/A | Y | N |
| 1 | Transmitter | Monthly | Check connection, clean, check battery voltage |  |  |  |  |
| 2 | Switch | Monthly | Check connection, clean, check power |  |  |  |  |
| 3 | Level Switch | Monthly | Proper operation, clean connection |  |  |  |  |
| 4 | Float Valve | Monthly | Function, seals on body, movement, clean strainer |  |  |  |  |
| 5 | Isolation Valve | Monthly | Check operation, Lubricant |  |  |  |  |
| 6 | Motion sensor | Monthly | Clean |  |  |  |  |
| 7 | Infrared sensor | Monthly | Clean |  |  |  |  |
| 8 | Beam Sensor | Monthly | Clean, local beam test |  |  |  |  |
| 9 | VESDA | Monthly | Check air flow, clean sample ports, conduct piping integrity smoke test, check power supply |  |  |  |  |
| 10 | CO Sensor | Monthly | Remove dust and test alarm |  |  |  |  |
| 11 | Window sensor | Monthly | Clean, check power |  |  |  |  |
| 12 | Occupancy sensor | Monthly | Clean, check power |  |  |  |  |
| 13 | Smoke & Fire sensor | Monthly | Proper operation, check alarm |  |  |  |  |
| 14 | Temperature sensor | Monthly | Check accuracy with temperature gauge |  |  |  |  |
| 15 | Solenoid valve | Monthly | Inspect for leakage, check wiring on coil, connection |  |  |  |  |
| 16 | Flow meter | Annually | Calibration, check connections, clean monitor |  |  |  |  |
| 17 | Photo electric sensor | Monthly | Clean eye, check power |  |  |  |  |
| 18 | Vibration sensor | Annually | Clean, check accuracy, check connections |  |  |  |  |
| 19 | Thermocouples | Annually | Clean, check position, check coupler |  |  |  |  |
| 20 | Humidity sensor | Monthly | Check sensor filter for dirt and dust |  |  |  |  |
| 21 | DDC controller | Monthly | Check calibration, check set points, check data base entries, check damper actuators |  |  |  |  |
| 22 | Current switch | Monthly | Check connectors, clean, inspect for damage, check for corrosion |  |  |  |  |
| 23 | Relay | Monthly | Check connectors, clean, inspect for damage, check for corrosion |  |  |  |  |
| 24 | Current Transformer | Monthly | Check primary current, check operation, rated load |  |  |  |  |
| 25 | Voltage transformer | Monthly | Check primary voltage, check operation, rated load |  |  |  |  |
| 26 | Smart meter | Annually | Check current, clean, any damage, check function, calibration |  |  |  |  |
| 27 | I/O modules | Annually | Check margins, clean, connections, calibrate |  |  |  |  |
| 28 | Damper | Monthly | Check linkage, freedom of movement, clean |  |  |  |  |
| 29 | Air Pressure sensor | Monthly | Visual checks, seals, connections |  |  |  |  |
| 30 | Alarms | Monthly | Alarm test, power supply |  |  |  |  |
| 31 | Controller screen | Monthly | Clean |  |  |  |  |
| 32 | O&Ms | Annually | Ensure documentation current, QA/QC |  |  |  |  |
| 33 | Power supplies | Monthly | Check all output voltages in specification |  |  |  |  |
| Additional Notes: |  |  |  |  |  |  |  |
| Work complete: Y/N |  |  | Signature: |  |  |  |  |
| Name: |  |  |  |  |  |  |  |