| Building NAME: |  | Reference No. |  |  |  | REV-00A |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Maintenance Checklist Electric Elevators |  |  |  |  |  | CHECKED SATISFACTORY |  |  |
|  |  |  |  |  |  |  | N/A | YES | NO |
|  | Elevators |  |  |  |  |  |  |  |  |
| 1 | General |  | Check all components are clean and are free from contamination/corrosion |  |  |  |  |  |  |
| 2 | Pit Areas |  | Check for excess oil/grease upon guides or base Ensure pit area is clear and drains clear Pit lighting operational |  |  |  |  |  |  |
| 3 | Buffers |  | Check condition |  |  |  |  |  |  |
|  |  |  | Check oil level |  |  |  |  |  |  |
|  |  |  | Ensure lubrication in operation |  |  |  |  |  |  |
|  |  |  | Check operation of proximity/limit switch |  |  |  |  |  |  |
|  |  |  | Check security of fixings |  |  |  |  |  |  |
| 4 | Brake |  | Check operation of braking system |  |  |  |  |  |  |
|  |  |  | Check for undue wear or defect |  |  |  |  |  |  |
|  |  |  | Ensure operating correctly |  |  |  |  |  |  |
| 5 | Drive motor |  | Check bearings for signs of wear (vibration/noise) |  |  |  |  |  |  |
|  |  |  | Ensure lubrication in place |  |  |  |  |  |  |
|  |  |  | Check condition of commutator or excessive brush debris |  |  |  |  |  |  |
| 6 | Gearbox |  | Check for excessive vibration/noise |  |  |  |  |  |  |
|  |  |  | Ensure lubrication in place |  |  |  |  |  |  |
| 7 | Sheave |  | Check condition of sheave and ropes alignment. Uneven wear may indicate incorrect rope tension |  |  |  |  |  |  |
| 8 | Overspeed Governor/Pulley |  | operational and fee to move |  |  |  |  |  |  |
|  |  |  | Check operation of limit switch |  |  |  |  |  |  |
|  |  |  | Check operation of overspeed / tension pulley |  |  |  |  |  |  |
| 9 | Controller |  | Check security of cabinet cooling fans unobstructed |  |  |  |  |  |  |
| 10 | Diverter Pulley |  | Check grooves for excessive wear |  |  |  |  |  |  |
|  |  |  | Check bearings for signs of wear (vibration/noise) |  |  |  |  |  |  |
|  |  |  | Ensure all guards in place |  |  |  |  |  |  |
|  |  |  | Ensure lubrication in place |  |  |  |  |  |  |
| 11 | Counterweight/Guides/Shoes |  | Ensure lubrication of guides |  |  |  |  |  |  |
|  |  |  | Check run by clearances |  |  |  |  |  |  |
|  |  |  | Check guides/rollers and shoes for excessive wear |  |  |  |  |  |  |
|  |  |  | Check security of all fixings |  |  |  |  |  |  |
| 12 | Electrical |  | Check electrical wiring and connections secure and attached |  |  |  |  |  |  |
|  |  |  | Carry out thermographic survey to check for overheating terminals/cables |  |  |  |  |  |  |
| 13 | Elevator Car |  | Check emergency lighting operational |  |  |  |  |  |  |
|  |  |  | Check auto dialer function |  |  |  |  |  |  |
|  |  |  | Check all panels are secure |  |  |  |  |  |  |
|  |  |  | Check maintenance panel is locked |  |  |  |  |  |  |
| 14 | Safety Equipment Overspeed Protection Devices |  | Check all parts free to operate and not obstructed |  |  |  |  |  |  |
|  |  |  | Ensure lubrication in place |  |  |  |  |  |  |
|  |  |  | Check security of all fixings |  |  |  |  |  |  |
|  |  |  | Check operation of limit switch |  |  |  |  |  |  |
| 15 | Suspension Ropes |  | Check for signs of damage, fraying of cores |  |  |  |  |  |  |
|  |  |  | Check for elongation |  |  |  |  |  |  |
|  |  |  | Check tension |  |  |  |  |  |  |
|  |  |  | Check for lubrication (if applicable to installation) |  |  |  |  |  |  |
| 16 | Suspension Rope Fixing and Anchorages |  | Check for signs of wear or damage |  |  |  |  |  |  |
|  |  |  | Check security of all fixings |  |  |  |  |  |  |
| 17 | Landing Apertures |  | Check operation of landing locks |  |  |  |  |  |  |
|  |  |  | Check door operation (juddering, sticking, alignment) |  |  |  |  |  |  |
|  |  |  | Check door guides free from debris detritus |  |  |  |  |  |  |
|  |  |  | Check door gaps |  |  |  |  |  |  |
|  |  |  | Check emergency unlocking devices |  |  |  |  |  |  |
|  |  |  | Check lubrication of moving items |  |  |  |  |  |  |
|  |  |  | Remove build up of grease dirt on rollers |  |  |  |  |  |  |
| 18 | Car Door |  | Check contacts for close |  |  |  |  |  |  |
|  |  |  | Check door running |  |  |  |  |  |  |
|  |  |  | Check door guides free from debris detritus |  |  |  |  |  |  |
|  |  |  | Check door gaps |  |  |  |  |  |  |
|  |  |  | Check lubrication of moving items |  |  |  |  |  |  |
|  |  |  | Check passenger door protection device |  |  |  |  |  |  |
| 19 | Floor Levelling |  | Ensure car levels to within prescribed limits |  |  |  |  |  |  |
| 20 | Level Limit Switch |  | Check Operation |  |  |  |  |  |  |
| 21 | Motor Time Limiter Device |  | Check Operation |  |  |  |  |  |  |
| 22 | Safety Devices |  | Check Operation |  |  |  |  |  |  |
|  |  |  | Check correct rating of fuse protection |  |  |  |  |  |  |
| 23 | Landing Indicators and Calls |  | Check operation of landing lights |  |  |  |  |  |  |
|  |  |  | Check audible signal operation |  |  |  |  |  |  |
|  |  |  | Check buttons operational and illuminated |  |  |  |  |  |  |
| 24 | Shaft Lighting |  | Check shaft lighting operational |  |  |  |  |  |  |
| No. | Reviewer's Comments |  |  |  | Resolution |  |  |  |  |