4-Wheel-Drive Vehicles – TYPE A SERVICE – Every 10,000KM

Change engine oil and oil filter element

Clean oil filter cap and crank case breather

Clean air cleaner element and renew air cleaner oil

Clean fuel sediment bowl

Clean and reset contact breaker points (petrol engine)

Clean and reset spark plugs (petrol engine)

Lubricate all grease points

Check oil levels in: a. Gearbox, hub reductions and differentials b. Steering swivel joints c. Steering box d. Fuel injection pump

Check fluid levels in: a. Brake and clutch master cylinders b. Windscreen washer bottle(s)

Lubricate with oil-can or grease: a. Throttle and accelerator linkages b. Door locks and hinges c. Bonnet and boot fastenings and locks

Check and rectify if necessary: a. Battery terminals cleanliness b. Battery clamps for tightness c. Fan belt tension (DO NOT OVERTIGHTEN) d. Engine idling speed e. Water and oil leaks f. Clutch and brake pedal clearances g. Body and spring U-bolts h. Tire pressures and wear i. Wheel nuts for tightness j. Prop-shaft bolts k. Wheel alignment and balancing

Notes: Do not overfill oil levels Remove oil drips and spills after topping up and lubricating Wipe away surplus grease and fingermarks from paintwork and interior

4-Wheel-Drive Vehicles – TYPE B SERVICE – Every 20,000KM

Type “A” plus:

- Replace fuel filters

Replace air cleaner element (instead of cleaning)

Replace contact breaker points (instead of cleaning and resetting)

Lubricate distributor

Replace spark plugs (instead of cleaning and resetting)

Check fluid levels in: Battery Radiator Check and rectify if necessary Suspension bushes Wheel bearing adjustment Engine and gearbox mounting Brake, clutch and fuel pipes Exhaust fittings for tightness Reset valve clearances

Check brake linings and drums, clean and report if worn

4-Wheel-Drive Vehicles – TYPE C SERVICE – Every 40,000KM

Type “B” plus:

1. Clean fuel tank

2. Replace distributor condenser

3. Drain and renew oil in hub reduction gears

4. Drain and renew oil in gearboxes

5. Drain and renew oil in differentials

6. Clean and repack front wheel bearings

7. Tires and tubes rotation

4-Wheel-Drive Vehicles – TYPE D SERVICE – Every 60,000KM

Type “B” plus:

- Drain and flush engine coolant and renew

Renew power steering fluid

Drain and renew brake fluid

Drain and renew clutch fluid

Check condition of tires and tubes – report if worn

Check condition of shock absorbers – report if worn

Check condition of wheel bearings – report if worn

4-Wheel-Drive Vehicles – TYPE E SERVICE – Every 100,000KM

Type “B” plus:

- Change fuel injector nozzle

Clean axle breathers

Replace timing belt